Checklist

Mixer

F8 Manual references

1	Plugin Mics

2	Turn Phantom power On for condenser mic channels, Boom. - 90 -

Keep Off for Radio Mic channels.

3	Turn Limiter On for all channels (on ZoomF8 “On Normal” is fine). - 83 -

4	Bring Faders up to just above zero (centre).

5	Set input levels (Trim or Gain). Average -12Db. - 28 -

6	Check/Set Sample Rate (48kHz) and Bit Depth (24bits) – 30 -

7	Name folders (if recorder allows it) – 55 -

8	Test Record

9	Engage EQ (High Pass Filter HPF) if required. - 82 -

Radio Mics

ew-100p G4 Manual references

1	Turn on the Receivers.

2	Scan bandwidth. Select: Set – Easy Setup – Scan New List – Current List – Bank ‘X’ – 	Channel ‘X’ – Set. (remember to have all actors on the same Bank different Channels).

3	Sync the Transmitter and Receiver to the same frequency. Select Sync with both units doors 	open facing each other. - 121 -

Alternatively tune it manually.  Advanced – Tune – Adjust the 	frequency and ‘Set’ to 	confirm each choice. - 100 -

4	Listen and check for a clean signal.